Section 7 Mycobacteria

7.1 Molecular diagnostics for identification and monitoring of mycobacterial infections

The major challenge for diagnosis of tuberculosis is the slow rate of growth of the organism in conventional and automated culture.  There are numerous examples of transmission of the infection among vulnerable patients as a result of failure to detect tuberculosis rapidly.    Stained smears of infected material (Ziehl-Neelsen or auramine) are, in contrast, rapid but lack both sensitivity and specificity. For many years, culture has been considered as the gold standard i.e., reference method for tuberculosis diagnosis.  Positive culture results may be available in as few as five days in patients with a high bacterial load, but the negative results (from liquid culture) can only be finally reported more than six weeks after sample receipt.  Drug susceptibility requires an additional week at a minimum and may require significantly longer time if the organism is in low concentration. Despite the disadvantage of being slow the limit of detection for culture is low translating into optimal sensitivity.  Consequently, it remains the cardinal diagnostic method.

There has been a longstanding endeavour to develop rapid molecular based assays.

7.1.1 Detection of Nucleic Acid Species

DNA

Multiple methods of detecting mycobacterial DNA in clinical samples have been reported.  The recent release of the GeneXpert® system for tuberculosis (Xpert MTB/RIF; Cepheid, Sunnyvale, CA) has raised considerable interest with regard to its potential use as a biomarker of treatment response. The results of recent clinical trials using this method for tuberculosis diagnosis are discussed in detail below.  However, it should be noted that older studies suggest that mycobacterial DNA remains in the sputum for a considerable period of time after initiation of therapy and this may compromise the value of this technique.

RNA

RNA can be used for detection and quantification of M. tuberculosis in clinical samples (Honeyborne et al., 2011).  Further details are described in section 7.1.4.

7.1.2 Cepheid GeneXpert

The Cepheid GeneXpert® System is a fully-automated and integrated system for Polymerase Chain Reaction (PCR)-based nucleic acid (DNA) testing.  There are 11 FDA-cleared assays and 12 CE-IVD marked assays that use the Xpert platform, including tests for enteroviral meningitis, methicillin-resistant Staphylococcus aureus (MRSA), Influenza A and B, group B Streptococcus, and anthrax, in addition to the assay for Mycobacterium tuberculosis and rifampin resistance detection.  The tuberculosis assay consists of a microfluidic cartridge with multiple chambers that contains all necessary reagents and diluents for sample processing, nucleic acid amplification, and detection of products.  The respiratory sample, after treatment with sample buffer, is added with a pipette into one chamber of the cartridge, the top is closed, and the cartridge is placed into the assay module in which all subsequent testing steps occur (see Figure 1).  The reaction tube, unlike the cylindrical shape of most PCR reaction tubes, is a flat, window-like shape, allowing efficient and rapid heating and cooling by the solid-state heating element present in the assay module.  Liquids move between chambers through the action of a rotary valve and a central syringe-like plunger.  The PCR reactants are contained in freeze-dried beads, which are dissolved only when the liquid enters the appropriate chamber.  Internal software controls monitor pressure, reagent presence, inhibition, and bead activity.  The ease of use allows non-technical workers to perform the assay with minimal training.  This concept has been validated in a study conducted by Boehme and others.

Prior to initiating the Xpert MTb/RIF assay, the respiratory sample is mixed with a sample reagent at twice the volume of the sputum and shaken vigorously; this liquefies mucus and is cidal for the mycobacteria.  Two mls of the suspension is added to the cartridge; this large sample volume increases the sensitivity of the assay.  A second factor in enhanced sensitivity is the action of a glass-bead enhanced sonication of the mycobacteria, which are fixed on a filter at the base of the cartridge, efficiently releasing DNA.  The DNA passes through the filter into the reaction buffer chamber.  A third factor that enhances assay sensitivity is the use of a hemi-nested PCR amplification protocol.  The first reaction amplifies the specific rpoB genetic sequence and the second PCR reaction amplifies each of 5 smaller internal fragment.  The 6-color molecular beacon assay was designed by Dr. David Alland and Dr. David Persing.  From start to finish, results are available within 2 hours of sample receipt, which includes approximately 5 minutes of hands-on manipulation.  The PCR amplifies an 81 base-pair region of the ribosomal polymerase B gene, which is unique to M. tuberculosis complex.  The majority of mutations that confer rifampin resistance are in this region.  Five molecular beacons, each with a different fluorophore, bind to independent regions of the amplicon.  A sixth target identifies the internal control, a Bacillus globigii spore which indicates any potential inhibition in the reaction.  Similar cycle threshold amplification of all five target sequences signifies a wild-type, rifampin-susceptible M. tuberculosis complex organism, whereas delayed or missing signals of one to three targets signifies one or more mutations conferring rifampin resistance.  The reaction is semi-quantitative, so that higher cycle threshold levels suggest lower numbers of organisms in the sample.  For the CE-IVD version, the results for M. tuberculosis detection are reported as “high, moderate, low, or very low.”  The quantitative estimation ability of the assay has been reported anecdotally in at least 6 publications, and Blakemore et al. have suggested an algorithm for incorporating the amount of inhibition into the result.

Several publications have described the development of the assay, sensitivity of the assay sensitivity of the assay in samples from children, and the biosafety features of the sample preparation and handling procedures.  The first large-scale assessment of the assay was conducted by the Foundation for Innovative New Diagnostics (FIND) in five sites with relatively high numbers of HIV-positive patients and multi-drug resistant TB (MDR-TB) patients: Peru, Azerbaijan, India, and South Africa.  Results from specimens collected from more than 1700 patients were evaluated and compared with the results of acid fast smear, liquid culture, and mycobacterial drug susceptibility tests.  A subsequent study by FIND evaluated the performance of the assay in a more rural setting with inexperienced laboratory workers.  Posters presented at several international meetings have also reported on use of the assay in large-scale testing situations (Naidoo, others).  At least one study showed that the assay failed to detect some positive samples that a laboratory-developed molecular assay was able to detect.  A number of studies have evaluated the use of the Xpert assay for samples other than respiratory secretions, with varying levels of success.

Several features of the Xpert® Mtb/RIF assay differ from existing molecular diagnostic methods.  The most commonly used nucleic acid amplification test (NAAT) in the United States is the GenProbe MTD assay, which targets and amplifies ribosomal RNA rather than DNA, but does not have a rifampin resistance detection component.  The GenProbe assay has been reported to be 97% sensitive and 96% specific for smear-positive samples, and 76% sensitive and 97% specific for smear-negative samples.  This test is highly complex, takes about 2 hours of hands-on time, and delivers results in 6 hours.  Roche also has two NAATs for M. tuberculosis, which are not currently available in the United States. Published results show 96% sensitivity and 74-83% specificity in smear-positive patient samples and 76% sensitivity and 97% specificity for smear-negative samples.  BD Probe-Tec shows similar results as the other two NAAT products in a meta-analysis based on 12 studies.  The ligase-chain reaction product from Abbott (10 studies) actually yielded slightly worse performance, with 96% sensitivity and 71% specificity for smear-positive samples and 57% sensitivity and 98% specificity for smear-negative samples and has since been withdrawn from the market.

In comparison with acid fast smear results, the Xpert Mtb/RIF assay is more sensitive and the results are delivered within the same time frame, i.e., approximately two hours.  All other types of methods require longer time periods to produce results, and in rural settings, many final results never reach the intended user. Training to perform the Xpert assay is more successful than training to read TB smears. Because extensive infrastructure, biosafety equipment, and special environments are not needed for performance of GeneXpert, the startup costs and continuing resources (including maintaining proficiency of workers) are lower than for development of liquid culture capability for a new testing setting. With regard to clinical endpoints, the implications of positive PCR results after or even during therapy have not yet been determined. Studies have shown that even if samples yield smear-negative results, it does not mean that the patient cannot transmit tuberculosis to others. Another recent study using a guinea pig sentinel system has confirmed that patients who are smear-negative may still be infectious, whereas those patients on effective therapy, despite having positive smears, are not likely to be infectious.  Moreover, patients with MDR TB being treated presumptively with conventional front-line TB regimens have been shown to be infectious in this model.

The Cepheid GeneXpert System (Xpert) is a fully automated and integrated system for Polymerase Chain Reaction (PCR)-based nucleic acid (DNA) testing.  Xpert targets the rpoB hotspot region of M tuberculosis DNA providing simultaneous speciation and identification of rifampicin resistance.  It requires minimal hands on time. The ease of use allows non-technical workers to perform the assay with minimal training.   This makes it an attractive approach in countries where it is difficult to recruit the experienced laboratory staff needed to perform smear and culture examination effectively.  It has already been shown to be more sensitive than conventional stained smear examination in clinical studies in high burden countries and has been implemented widely in South Africa replacing smear in the national tuberculosis programme.  The full test characteristics are still to be evaluated in communities where tuberculosis is relatively uncommon but here there is an advantage for laboratories who have a low throughput of tuberculosis tests in which there is a risk that laboratory skills will have declined.

One limitation of DNA tests like GeneXpert® is that they potentially detect both dead and viable organisms at the same level of sensitivity.  It should be noted that older studies suggest that mycobacterial DNA remains in the sputum for a considerable period of time after initiation of therapy and this may compromise the value of this technique as a way of monitoring therapy.

The GeneXpert® Mtb/RIF assay could be used to determine the infection status of patients rapidly for entry into a clinical trial.  The semi-quantitative nature of the results suggests the potential to use the assay to monitor a patient’s response to treatment (see below).  A current clinical trial comparing patients treated during the intensive phase of therapy with either rifampin or rifapentine (CDC-TB Trials Consortium Study 29X trial) has been expanded to include evaluation of the Xpert Mtb/RIF assay results at several time points to determine the feasibility of this approach.  Poor performance of GeneXpert® for monitoring sequential data has been confirmed in a recent publication from the REMoxTB consortium that that GeneXpert® ct values responded less rapidly than smear.

7.1.3 Hain system

An alternative to GeneXpert® is the line probe assay such as the Hain assay which detects amplimers specific to M. tuberculosis and some of the drug resistance genes by PCR and hybridisation of the amplimers to a nitrocellose strip.  Although technically straightforward to perform care must for the test steps to be performed consistently if the results are to be reproducible.  The test strips, compared with control pattern, can be photographed to maintain a record as the bands fade on storage.  Isoniazid rifampicin resistance are determined using the standard strips, and extended resistance testing with Hain S+ is available giving it an advantage of coverage over the geneXpert system.  This system has proved valuable in the REMoxTB clinical trial in areas of high MDRTB prevalence.

Follow up visit with the patient complaining of symptoms returning

Figure 7.1.1 An example of Hain test results in a patient being treated for tuberculosis.  Screening visit

7.1.4 Molecular Bacterial Load Assay

The Molecular Bacterial Load (MBL) assay monitors the molecular load of TB cells in clinical or model samples and hence provides accurate information on bacterial response (decline) to antimicrobial treatment.

Introduction

The MBL assay detects M. tuberculosis 16S ribosomal RNA, which is a relatively stable RNA species and after its extraction from sputum can provide the information on bacterial viable counts. The assay involves internal control that monitors for the RNA extraction efficiency and amplification performance. Internal control (IC) is an exogenous RNA template added to each specimen before the RNA extraction. The IC can be prepared as described in Honeyborne I. et al., J Clin Microbiol, 2011.  The TB and IC detection is based on a quantitative duplex real-time PCR and the identification of a specific fluorescent signal.

Materials and Methods

General consumables

- sterile DNAse and RNAse-free microtubes, 1.5 ml

- pipettes and sterile pipette tips, DNAse and RNAse-free, range: P1000, P200, P10, P2

- sterile disposable universal tubes

- PCR reaction tubes – use accordingly with the PCR instrument and the total number of samples to be analysed. For RotorGene Q (Qiagen) use:

- Single 0.2 ml PCR optical thin wall flat cap microtubes (any supplier), for 32-well rotor

- Strip tubes and caps, 0.1 ml (Qiagen), for 72-well rotor

- Rotor-Disc 100 with heat-seal film (Qiagen), for 100-well rotor

Instruments

- Centrifuge and benchtop centrifuge

- Thermoshaker

- Homogenizer (Fastprep, MP Biomedical or Precellys, Peqlab)

- Real-time PCR thermocycler (RotorGene Q, Qiagen)

7.1.4.1 RNA Preservation

The integrity of TB RNA in sputum samples can be maintained by preservation of the sputum in GTC solution immediately after expectoration, and storing at -80°C until used for the RNA extraction.

RNA preservation solution: 4 M GTC (Promega), 0.1 M HCl (pH 7.0) and 1% -mercaptoethanol

7.1.4.2 RNA extraction

We recommend using of either of two simple and rapid extraction procedures for the total RNA extraction. By this procedure, samples are lysed and homogenized in the presence of guanidinium isothiocyanate, a chaotropic salt capable of protecting the RNA from endogenous RNases. After lysis and homogenization and alcohol precipitation, the sample is processed through a spin column with silica-based membrane to which the RNA binds. Any impurities are removed by subsequent washing and the resulting RNA is eluted into water or a buffer.

For FastRNA Pro Blue kit (MP Biomedicals follow the manufacturer’s procedure.

For Purelink RNA (Ambion) extract the RNA according to following procedure:

Reagents

RNA PureLink kit (Ambion)

TURBO DNase I digest kit (Ambion)

Glass beads, 0.1 mm (e.g. PEQlab or Sigma), 0.65 g per 2 ml homogenisation tube (weighed in homogenisation 2 ml tubes).

β-mercaptoethanol

70% ethanol prepared fresh before the extraction

RNAse free water

IC RNA, 50 ng per RNA preparation

Procedure

Before starting, prepare fresh Lysis Buffer containing Purelink Lysis Buffer, 1% -mercaptoethanol and internal control (IC) RNA for each purification procedure (the internal control should be approximately constant for all the RNA readings). I.e. for 600 l of Lysis Buffer per each sample, add into 594 ml of Purelink Lysis Buffer a volume of 6 μl –mercaptoethanol.

- Thaw the sputum samples in GTC on ICE and keep the tubes on ice up to the point of adding the lysis buffer.

- Following thawing, add into each sputum sample in GTC 50 ng of internal control.

- Spin the tubes at 2,000 x g for 30 min at room temperature.

- Discard the supernatant and add 600 µl Lysis Buffer (supplemented with β-mercaptoethanol) to the pellet and agitate to detach the pellet from the tube. Work at room temperature further on.

- Transfer the suspension into homogenisation tube and process in a homogeniser. FastPrep setting 6, 40 s or Precellys 30 s at 6000 rpm.

- Spin the homogenisation tubes at 12000 x g for 5 min at room temperature.

- Remove 400 µl of the homogenate without touching or transferring any glass beads, and add the homogenate to a 1.5 ml RNase free microtube with one volume (400 µl) of 70% ethanol. Mix well by vortexing. Note: If part of the sample was lost during homogenization, adjust the volume of ethanol accordingly. Briefly spin to remove any drops from the lid.

- Transfer up to 500μl of the sample (including any remaining precipitate) to the Spin Cartridge inserted in a Collection Tube.

- Centrifuge at 12,000 × g for 15 seconds at room temperature. Discard the flow-through, and reinsert the Spin Cartridge in the same Collection Tube.

- Repeat until the entire sample is processed.

- Add 700μl Wash Buffer I to the Spin Cartridge containing the bound RNA. Centrifuge at 12,000 × g for 15 seconds at room temperature. Discard the flow-through and the Collection Tube. Insert the Spin Cartridge into a new Collection Tube.

- Add 500 μl Wash Buffer II (with ethanol added) to the Spin Cartridge. Centrifuge at 12,000 × g for 15 seconds at room temperature. Discard flow-through and reinsert the Spin cartridge into the same collection tube.

- Repeat Step the previous washing step once to remove any residual ethanol.

- Centrifuge the Spin Cartridge at 12,000 x g for 1 minute to dry the membrane with bound RNA. Discard Collection Tube and insert the Spin Cartridge into a Recovery Tube.

- Add 50µl (or 30μl–100μl) RNase–Free Water to the centre of the Spin Cartridge.

- Incubate at room temperature for 1 minute.

- Centrifuge Spin Cartridge and Recovery Tube for 1 minute at ≥12,000 x g at room temperature.

- Store purified RNA at –80°C or proceed directly to DNase treatment.

DNase treatment

- Add 0.1 volume of 10× Turbo DNase buffer (5 l) and 1 l (2U) Turbo DNase to each RNA sample. Mix gently.

- Incubate at 37°C, 30 min, 1000 rpm.

- Add again 1 l (2U) Turbo DNase and incubate for another 30 min (can be longer).

- Add 2 l homogenized (thawed & vortexed) DNase inactivation reagent. Incubate at room temperature for 5 min and up to 1000 rpm.

- Centrifuge at 10,000 × g for 1.5 min and transfer the RNA (leaving the pellet in the tube) into a new RNase free tube.

- Store the RNA at –80°C.

7.1.4.3 Quantitative RT-PCR reaction.

A one step RT real-time PCR is used for the RNA quantification. In one step and one reaction tube, the RNA is reverse transcribed to cDNA and this serves as template in the real-time PCR.

Considerations before start

Use RT-PCR mix without any Rox dye (e.g. QuantiTect Multiplex RT-PCR NR Kit, cat. no. 204843, Qiagen) for the use with RotorGeneQ, ViiA7 and CFX96 instruments, respectively. The ABI7300 instrument requires the use of a mastermix containing the Rox dye (e.g. QuantiTect Multiplex RT-PCR Kit, cat. no. 204643, Qiagen).

Aliquots of 500 l multiplex NR RT-PCR mix (Qiagen) are prepared and stored at -20°C. Paired forward and reverse primers are mixed in a single tube to the final concentration of 20 M and stored at -20°C. Probes are diluted to 20 M and aliquots are stored in -20°C. Aliquots of RT-PCR mix and probe are for single use only.

All oligonucleotides are of HPLC quality.

Water is of PCR grade and RNase-free.

Work on ice when preparing the PCR mastermix and individual reactions. Thaw and keep the RNA on ice.

Use the RNA template neat and as 1/10 dilution

Every sample, including the neat and diluted RNA and the dilutions for the standard curve, is analyzed in duplicate.

Reagents

QuantiTect Multiplex RT-PCR NR Kit

RNAse free water

Primers for TB and IC

Fluorescently labeled TaqMan probes for TB (FAM labelled) and IC (HEX labelled)

Hazards and Personal Protective Equipment

Lab coats and nitrile gloves must be worn at all times during the practical

Procedure

- Prepare the PCR mastermix for all samples to be analysed according to Table 1 and calculate for 10% extra volume in order to prevent pipetting errors. Vortex the mastermix and briefly spin down.

- Distribute 8 l of the mastermix into each reaction tube.

- Add 2l of RNA template (neat and diluted) into each tube. Add 2l TB DNA (positive control) and 2l nuclease-free water (negative control).  Analyze each sample in duplicate.

- Run the PCR in the real-time PCR instrument, set the thermal cycling conditions as detailed in Table 2.

Standard curve construction.

Use the RNA extracted from a standard material - M. tuberculosis culture with estimated concentration of 108 CFU/ml or higher. In detail, dilute the extracted RNA decimally to create a series of standards. Prepare 7 decimal dilutions and use them together with the neat RNA to construct the standard curve for PCR quantification.

The standard curve can be prepared in a separate run for the use with RotorGene Q and it can be further incorporated in data analysis of samples with unknown bacterial load.

The PCR efficiency can be evaluated by the parameters of standard curve. The equation for an ideal standard curve and 100% amplification efficiency (E=1) is:

Ct = slope×Log(concentration) – intercept

or

Ct = -3.32×Log(concentration) – intercept

Aim for the efficiency of 90%-100%, i.e. E=0.9 to 1.0. The efficiency can be calculated from the slope of the standard curve using the equation:

E=10-1/-3.32-1

| Reagent | Stock concentration | Reaction concentration | Volume per reaction (l) | Volume per n reactions (l) |
| --- | --- | --- | --- | --- |
| RNA template |  |  | 2 |  |
| IC Forward + Reverse primer | 20 M | 200 nM | 0.1 |  |
| TB 16S Forward + Reverse primer | 20 M | 200 nM | 0.1 |  |
| IC probe (HEX-BHQ1) | 20 M | 200 nM | 0.1 |  |
| TB 16S probe (FAM-BHQ1) | 20 M | 200 nM | 0.1 |  |
| Quantitect RT-mix | 2× | 1× | 5 |  |
| Quantitect RT enzyme |  |  | 0.1 |  |
| RNase free water |  |  | 2.5 |  |
| Total |  |  | 10 |  |

Table 7.1.4.1 PCR reaction composition.

Table 7.1.4.2 Thermal cycling programme.

| Hold | 50°C | 20 min |  |
| --- | --- | --- | --- |
| Hold | 95°C | 15 min |  |
| 40 cycles | 94°C | 45 s |  |
|  | 60°C | 45 s | Acquiring fluorescent signal for FAM and HEX |

Data analysis

The TB-specific amplification is detected by the fluorescent signal in the FAM channel. The internal control amplification signal is detected in VIC (or HEX) channel.

Set the fluorescence threshold.

Import the standard curve for TB and IC. Using the instrument’s software, determine the CT values and sample concentrations.

Calculate the TB concentration in sputum.

7.1.4.4 Additional reading related to molecular diagnosis of M. tuberculosis

Ahmad S, Al-Mutairi NM, Mokaddas E. Comparison of performance of two DNA line probe assays for rapid detection of multidrug-resistant isolates of Mycobacterium tuberculosis. Indian J Exp Biol 2009;47:454-62.

Armand S, Vanhuls P, Delcroix G, Courcol R, Lemaitre N. Comparison of the Xpert MTB/RIF test with an IS6110-TaqMan real-time PCR assay for direct detection of Mycobacterium tuberculosis in respiratory and nonrespiratory specimens. J Clin Microbiol 2011;49:1772-6.

Banada PP, Sivasubramani SK, Blakemore R, et al. Containment of bioaerosol infection risk by the Xpert MTB/RIF assay and its applicability to point-of-care settings. J Clin Microbiol 2010;48:3551-7.

Blakemore R, Nabeta P, Davidow AL, et al. A Multi-Site Assessment of the Quantitative Capabilities of the Xpert(R) MTB/RIF Assay. Am J Respir Crit Care Med 2011.

Blakemore R, Story E, Helb D, et al. Evaluation of the analytical performance of the Xpert MTB/RIF assay. J Clin Microbiol 2010;48:2495-501.

Boehme CC, Nicol MP, Nabeta P, et al. Feasibility, diagnostic accuracy, and effectiveness of decentralised use of the Xpert MTB/RIF test for diagnosis of tuberculosis and multidrug resistance: a multicentre implementation study. Lancet 2011;377:1495-505.

Boehme CC, Nabeta P, Hillemann D, et al. Rapid molecular detection of tuberculosis and rifampin resistance. N Engl J Med;363:1005-15.

Ciftci IH, Aslan MH, Asik G. [Evaluation of Xpert MTB/RIF results for the detection of Mycobacterium tuberculosis in clinical samples]. Mikrobiyol Bul 2011;45:43-7.

El-Hajj HH, Marras SA, Tyagi S, Kramer FR, Alland D. Detection of rifampin resistance in Mycobacterium tuberculosis in a single tube with molecular beacons. J Clin Microbiol 2001;39:4131-7.

Friedrich SO, Venter A, Kayigire XA, Dawson R, Donald PR, Diacon AH. Suitability of Xpert MTB/RIF and Genotype MTBDRplus for Patient Selection for a Tuberculosis Clinical Trial. J Clin Microbiol 2011;49:2827-31.

Sven O Friedrich, Andrea Rachow, Elmar Saathoff , Kasha Singh, Chacha D Mangu, Rodney Dawson, Patrick P J Phillips, Amour Venter, Anna Bateson, Catharina C Boehme, Norbert Heinrich, Robert D Hunt, Martin J Boeree, Alimuddin Zumla, Timothy D McHugh, Stephen H Gillespie, Andreas H Diacon, Michael Hoelscher, on behalf of the Pan African Consortium for the Evaluation of Anti-tuberculosis Antibiotics (PanACEA) Assessment of the sensitivity and specificity of Xpert MTB/RIF assay as an early sputum biomarker of response to tuberculosis treatment Lancet Respiratory Medicine 2013; 1: 462–70

Helb D, Jones M, Story E, et al. Rapid detection of Mycobacterium tuberculosis and rifampin resistance by use of on-demand, near-patient technology. J Clin Microbiol;48:229-37.

Helb D, Jones M, Story E, et al. Rapid detection of Mycobacterium tuberculosis and rifampin resistance by use of on-demand, near-patient technology. J Clin Microbiol 2009;48:229-37.

Hillemann D, Rusch-Gerdes S, Boehme C, Richter E. Rapid molecular detection of extrapulmonary tuberculosis by the automated GeneXpert MTB/RIF system. J Clin Microbiol 2011;49:1202-5.

Honeyborne I, McHugh TD, Phillips PP, Bannoo S, Bateson A, Carroll N, Perrin FM, Ronacher K, Wright L, van Helden PD, Walzl G, Gillespie SH. Molecular bacterial load assay, a culture-free biomarker for rapid and accurate quantification of sputum Mycobacterium tuberculosis bacillary load during treatment. J Clin Microbiol. 2011 Nov;49(11):3905-11.

Ioannidis P, Papaventsis D, Karabela S, et al. Cepheid GeneXpert MTB/RIF Assay for Mycobacterium tuberculosis Detection and Rifampin Resistance Identification in Patients with Substantial Clinical Indications of Tuberculosis and Smear-Negative Microscopy Results. J Clin Microbiol 2011;49:3068-70.

Kennedy N, Gillespie SH, Saruni AO, et al. Polymerase chain reaction for assessing treatment response in patients with pulmonary tuberculosis. J Infect Dis 1994;170:713-6.

Ling DI, Flores LL, Riley LW, Pai M. Commercial nucleic-acid amplification tests for diagnosis of pulmonary tuberculosis in respiratory specimens: meta-analysis and meta-regression. PLoS One 2008;3:e1536.

Lawn SD, Mwaba P, Bates M, et al. Advances in tuberculosis diagnostics: the Xpert MTB/RIF assay and future prospects for a point-of-care test. The Lancet Infectious Diseases 2013; 13: 349–61.

Marlowe EM, Novak-Weekley SM, Cumpio J, et al. Evaluation of the Cepheid Xpert MTB/RIF assay for direct detection of Mycobacterium tuberculosis complex in respiratory specimens. J Clin Microbiol 2011;49:1621-3.

Miller MB, Popowitch EB, Backlund MG, Ager EP. Performance of Xpert MTB/RIF RUO Assay and IS6110 Real-Time PCR for Mycobacterium tuberculosis Detection in Clinical Samples. J Clin Microbiol 2011.

Nicol MP, Workman L, Isaacs W, et al. Accuracy of the Xpert MTB/RIF test for the diagnosis of pulmonary tuberculosis in children admitted to hospital in Cape Town, South Africa: a descriptive study. Lancet Infect Dis 2011.

Picard FJ, Gagnon M, Bernier MR, et al. Internal control for nucleic acid testing based on the use of purified Bacillus atrophaeus subsp. globigii spores. J Clin Microbiol 2009;47:751-7.

Rachow A, Zumla A, Heinrich N, et al. Rapid and accurate detection of Mycobacterium tuberculosis in sputum samples by Cepheid Xpert MTB/RIF assay--a clinical validation study. PLoS One;6:e20458.

Vadwai V, Boehme C, Nabeta P, Shetty A, Alland D, Rodrigues C. Xpert MTB/RIF: a new pillar in diagnosis of extrapulmonary tuberculosis? J Clin Microbiol 2011;49:2540-5.

7.2 IS 6110 DNA fingerprinting overview

DNA fingerprinting of Mycobacterium tuberculosis has been shown to be a powerful epidemiological tool. Standardisation of this technique was proposed in 1993 by Van Embden et al to exploit variability in both the number and genomic position of the insertion site (IS) 6110 (IS6110) to generate strain-specific patterns. The insertion sequence IS6110 is found in almost all isolates of M tuberculosis and is present in one to approximately 26 copies. The variability in the position of these insertion sequences provides the basis of a Restriction Fragment Length Polymorphism (RFLP) typing method. A standardised method permits results to be compared between different laboratories and therefore allows investigation of the international transmission of tuberculosis and identification of specific strains with unique properties such as high infectivity, virulence, or drug resistance, as well as monitoring cross contamination (Ruddy et al., 2002).

7.3 Repeat based typing: Variable Number Tandem Repeats (VNTR) overview

The IS6110 fingerprinting technique is based on electrophoretic separation followed by probing of largely intact whole genomic DNA, it is a relatively cumbersome method requiring a high concentration of high quality DNA that must be isolated from pure cultures.  It is also cumbersome for large numbers of strains and requires experienced staff both to ensure DNA quality and to perform the electrophoresis.  Standardisation between gels can be very challenging when large numbers of samples are to be compared.  Importantly, a significant number of strains of M. tuberculosis (5-40% depending on location) carry few or no copies of IS6110, which renders this method of little value.

More rapid alternatives to IS6110 fingerprinting have been developed. Spoligotyping has been used for a number of years (Goyal et al, 1997).  In 1998 Frothingham et al introduced Variable Number Tandem Repeat (VNTR) analysis and Philip Supply introduced a newer version called Mycobacterial Interspersed Repeat Unit (MIRU) analysis in 2000.  Both of these are methods are PCR-based which means they can be performed from broth cultures, such as BACTEC/MGIT, as soon as they are positive. It is important to remember that both VNTR and MIRU analysis are based on the same or very similar elements (figure 2.). Indeed, some of the elements overlap in the different schemes. They are highly reproducible and produce digital results that are easier to compare than the IS6110 patterns (figure 3). The M. tuberculosis genome contains 41 loci with direct tandem repeats of 50-70 base pairs (figure 4). The number of repeats per locus varies between strains and strains can be typed based on the number of repeats per locus. Variation in the number of repeats is thought to be due to strand slippage during replication (figure 5). Variation in the IS6110/MIRU patter occur at different rates depending on the differing molecular clock (McHugh et al.,  2005) VNTR variation is associated with function in various bacterial species and includes surface protein variation/regulation and phase variation or ‘switching’:

# Surface protein variation/regulation

- Opa genes

  - Neisseria gonorrhoeae

- Porin gene regulation

  - Neisseria meningitidis

- Emm gene

  - Group A streptococci

- Sheath protein

  - Bacillus anthracis

# Phase variation

- Lipopolysaccharide

  - Haemophilus influenzae

  - Neisseria gonorrhoeae

- Fimbriae

  - Haemophilus

IS6110

‘blue’ labelled fragments

IS6110

Frequent cutter (TaqI)

Genomic DNA

# Figure 7.3.1 VNTR structure and variation in repeat copy number between strains. This region can be amplified by PCR using primers (arrows) outside of the repeat region and the size of the PCR product, shown above each example, corresponds to the number of repeats.  So, for MIRU 4 (the number refers to the specific locus, not to the number of alleles) the allele numbers would be 1, 2, 3, and 6 for these examples. Combining the results for all 12 MIRU loci yields a 12-digit MIRU designation.

No. of copies

Size (bp)

MIRU

2

2

3

3

5

2

4

3

2

2

3

2

220

226

264

226

303

414

287

365

168

201

332

283

40

39

31

27

26

24

23

20

16

10

04

02

MIRU pattern:	232234253322

# Figure 7.3.2 Example of digital result for MIRUs.

# Figure 7.3.3 MIRU Locations in the Mtb genome (taken from Supply et al., 2000)

Figure 7.3.4 Genesis of Repeats in VNTRs. The peculiar tertiary structure of repetitive DNA allows mis-matching of neighbouring repeats, and, depending on the strand orientation, repeats can be inserted or deleted during replication.

7.3.1 Sizing of VNTR/MIRU loci

# Overview

As mentioned in the previous section, all VNTR methods are based on PCR amplification of the repeats and the flanking DNA regions, followed by size separation of those products using various methods. Deducing the number of repeats in VNTR PCR (essentially by sizing the PCR products generated) can be executed in different ways. The method used in many laboratories that can be performed without specialised instrumentation is agarose gel electrophoresis (included in workshop practical). This method does not require PCR primers to be labelled and does not require specialised software/instrumentation expertise although it can be relatively time consuming. The second method is fragment sizing using capillary electrophoresis (automated sequencer) instrumentation (figure 7.3.5). In this case, several VNTR loci can be amplified at once in the same PCR, as the capillary electrophoresis instrument is capable of recognising different fluorophores labelling the different loci. These are incorporated into the product during PCR by the use of labelled specific primers. The labelled products are then separated by capillary electrophoresis and compared against a known size standard and product size automatically called following the generation of a size curve (see figure 7.3.5 below). Products are usually sized within 1-2 bp using this method although this level of precision is not always necessary.

Figure 7.3.5 Multiplexed fragment sizing using products labelled with different fluorophores (‘blue’ and ‘green’) against a known size standard (in ‘red’).

# 7.3.2 VNTR/MIRU PCR Practical

# 7.3.2.1 DNA extraction

# Principle

# For the purposes of safety, we will extract DNA from M. smegmatis, a non-tuberculosis Mycobacterium species. We will use QIAamp, DNA Mini Kit, Qiagen and a modified extraction protocol. You will use M. tuberculosis DNA for the MIRU-VNTR.

Hazards and Personal Protective Equipment

Lab coats and nitrile gloves must be worn at all times during the practical

Proteinase K – Irritant, skin and respiratory sensitizer.

Buffer AW1 – Harmful, Irritant, Respiratory sensitizer.

Buffer AL-Skin and Eye, Irritant, Skin sensitizer

Ethanol – Highly flammable/Irritant

Protocol:

- Pellet 1 ml of fresh exponential culture at 10000 g for 10 min. Discard the supernatant.

- Suspend bacterial pellet in 180 μl of 20 mg/ml lysozyme in 20 mM Tris·HCl, pH 8.0; 2 mM EDTA; 1.2% Triton. Incubate for a minimum of 30 min) at 37°C, 1000 rpm.

- Add 20 μl proteinase K and incubate at 56 °C, 1000 rpm for a minimum of 30 min.

- Add 4 μl RNase A and incubate at room temperature for 2 min.

- Add 200 μl buffer AL, mix by vortexing.  Incubate at 70°C, 1000 rpm for 10 min and then for a further 15 min at 95°C.

- Briefly centrifuge the 1.5 ml microcentrifuge tube to remove drops from the inside of the lid.

- Add 200 μl ethanol (96–100%, ice-cold) to the sample, and mix by pulse-vortexing for 15 s. After mixing, briefly centrifuge the 1.5 ml microcentrifuge tube to remove drops from inside the lid. It is essential that the sample, Buffer AL, and ethanol are mixed thoroughly to yield a homogeneous solution. A white precipitate may form on addition of ethanol. It is essential to apply all of the precipitate to the QIAamp Mini spin column. This precipitate does not interfere with the QIAamp procedure or with any subsequent application. Do not use alcohols other than ethanol since this may result in reduced yields.

- Carefully apply the mixture (approximately 600 l, including the precipitate) to the QIAamp Mini spin column (in a 2 ml collection tube) without wetting the rim. Close the cap, and centrifuge at 6000  g (8000 rpm) for 1 min. Place the QIAamp Mini spin column in a clean 2 ml collection tube (provided), and discard the tube containing the filtrate. Close each spin column to avoid aerosol formation during centrifugation. It is essential to apply all of the precipitate to the QIAamp Mini spin column. Centrifugation is performed at 6000  g (8000 rpm) in order to reduce noise. Centrifugation at full speed will not affect the yield or purity of the DNA. If the solution has not completely passed through the membrane, centrifuge again at a higher speed until all the solution has passed through.

- Carefully open the QIAamp Mini spin column and add 500 μl Buffer AW1 without wetting the rim. Close the cap, and centrifuge at 6000 x g (8000 rpm) for 1 min. Place the QIAamp Mini spin column in a clean 2 ml collection tube (provided), and discard the collection tube containing the filtrate.*

- Carefully open the QIAamp Mini spin column and add 500 μl Buffer AW2 without wetting the rim. Close the cap and centrifuge at full speed (20,000 x g; 14,000 rpm) for 3 min.

- Place the QIAamp Mini spin column in a new 2 ml collection tube (not provided) and discard the old collection tube with the filtrate. Centrifuge at full speed for 1 min. This step helps to eliminate the chance of possible Buffer AW2 carryover.

- Place the QIAamp Mini spin column in a clean 1.5 ml microcentrifuge tube (not provided), and discard the collection tube containing the filtrate. Carefully open the QIAamp Mini spin column and add 50 μl Buffer AE (or water). Incubate at room temperature for 1 min, and then centrifuge at 6000 x g (8000 rpm) for 1 min.

- Repeat previous step, i.e. elute the DNA once again with 50 μl Buffer AE (or water). Incubate at room temperature for 1 min, and then centrifuge at 6000 x g (8000 rpm) for 1 min.

- Store eluted DNA at -20°C.

- The quality and quantity of extracted DNA can be checked on NanoDrop and by electrophoresis in agarose gel.

# 7.3.2.2 MIRU-VNTR PCR

You have been provided with DNAs extracted from three M. tuberculosis isolates. You have also been provided with PCR reagents and primers for three of the current panel of 24 VNTR/MIRU loci (MIRU10, MIRU24 and MIRU26). Method and primers are according to Allix et al, Proposal for standardization of optimised mycobacterial interspersed repetitive unit-variable number tandem repeat typing of Mycobacterium tuberculosis. 2006 J Clin Microbiol. 44:4498-4510.

Hazards and Personal Protective Equipment

Lab coats and nitrile gloves must be worn at all times during the practical

2% E gels (contain ethidium bromide) – Toxic/Mutagen

Protocol

- Label three sterile 1.5 ml microtubes, one for each MIRU primer pair (one for MIRU10, one for MIRU24 and one for MIRU26), Label 0.2 ml PCR tubes (running each sample in duplicate) for all samples to be analysed three VNTR/MIRU PCR mixtures.

- Prepare PCR mixtures for each primer pair as in the table below.

| Reagent | Stock concentration | Reaction concentration | Volume per reaction (l) | Volume per n reactions (l) |
| --- | --- | --- | --- | --- |
| DNA template |  |  | 2 |  |
| Forward primer | 10 M | 200 nM | 0.5 |  |
| Reverse primer | 10 M | 200 nM | 0.5 |  |
| Sigma PCR mix | 2× | 1× | 12.5 |  |
| Sterile distilled water |  |  | 9.5 |  |
| Total |  |  | 25 |  |

Table 7.3.2.2.1 MIRU-VNTR PCR reaction composition.

- Dispense each VNTR/MIRU mastermix by 23 l into each 0.2 ml PCR tube and add 2 l of DNA sample into each tube.

- Carry out the PCR at following cycling conditions:

95°C, 12 min (important for a GC-rich template)

94°C, 30 s 	]

54°C, 1 min 	] 40 cycles

72°C, 2 min 	]

72°C, 7 min

# 7.3.2.3 Agarose gel electrophoresis of MIRU/VNTR  PCR products

Principle

PCR products will be separated according to their size and DNA negative charge in agarose gel electrophoresis. For ease of use we are going to size the products generated in the PCR by running them out on a ready-made E-gel (for product information see Section 4.0 Appendix 1) with a 1kb ladder size marker to compare the sizes of the products generated by our test DNAs D1-D3 for the three VNTR loci.

Protocol

- Plug the PowerBase™ into an electrical outlet.

- Open the package containing the gel (with the comb in place) into the apparatus right edge first. Press firmly at the top and bottom to seat the gel in the base. You should hear a snap when it is in place. The Invitrogen logo should be located at the bottom of the base, close to the positive pole. A steady, red light will illuminate when the E-Gel is correctly inserted (Ready Mode).

- Press and hold either button until the red light turns to a flashing green light. This indicates that the 2 minute pre-run has started.

- At the end of the pre-run the current will automatically shut off. The flashing green light will change to a flashing red light and the PowerBase™ will beep rapidly.

- Press and release either button to stop the beeping. The light will change from a flashing red light to a steady red light.

- Remove the comb from the gel using both hands to lift the comb gently by rolling the comb slowly towards you. Be careful to pull the comb up straight from both sides and do not bend the comb. Remove any excess fluid with a pipette.

- All wells in the gel must contain 20ul of sample or water. Load 5ul of 1kb ladder in the first well. Top up this well with 15ul of water. Load 5ul of each VNTR PCR product and the control PCR products except for one locus control as there are only 12 wells in the gel. Top up these wells with 15ul water.

- On the PowerBase™ choose a 30 minute run by pressing and releasing the 30 min button. The light will change to a steady green light. The end of the run is signalled by a flashing red light and a rapid being.

- Press and release either button to stop the beeping. The light will turn to a steady red light.

- Remove the gel from the power unit and analyze your results on a UV transilluminator.

Data interpretation

The 1kb ladder sizes are as follows:

# Figure 7.3.2.3.1 VNTR calculation using agarose gel electrophoresis

A simple calculation of fragment size minus the lengths of the forward and reverse primers and regions flanking the VNTR, divided by the length of one individual VNTR unit will give you the number of VNTR units for the isolate. A VNTR/MIRU sizing table is also shown for convenience (Table 7.3.2.3.1). Determine the sizes of the products and then calculate the number of repeats per product. An example is shown below

Is there a variation in copy number at each of the three loci for each DNA?

Example using MIRU4 (aka ETRD):

Figure 7.3.2.3.2  Example of MIRU/VNTR sizing. Lane 1 = 100 base pair (bp) marker, increasing in 100 bp increments (NB your marker is different to this picture).

Flanking region of PCR product = 141

Repeat = 77 bp

Table 7.3.2.3.1 VNTR/MIRU sizing table in base pairs

| Allele | MIRU 02 | MIRU 04 | MIRU 10 | MIRU 16 | MIRU 20 | MIRU 231 | MIRU 232 | MIRU 24 | MIRU 261 | MIRU 262 | MIRU 27 | MIRU 31 | MIRU 39 | MIRU 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 402 | 175 | 482 | 565 | 437 | 558 | 150 | 395 | 461 | 285 | 498 | 492 | 540 | 354 |
| 1 | 455 | 252 | 537 | 618 | 514 | 608 | 200 | 447 | 512 | 336 | 551 | 545 | 593 | 408 |
| 2 | 508 | 329 | 590 | 671 | 591 | 661 | 253 | 501 | 563 | 387 | 604 | 598 | 646 | 462 |
| 3 | 561 | 406 | 643 | 724 | 668 | 714 | 306 | 555 | 614 | 438 | 657 | 651 | 699 | 516 |
| 4 | 614 | 483 | 696 | 777 | 745 | 767 | 359 | 609 | 665 | 489 | 710 | 704 | 752 | 570 |
| 5 | 667 | 560 | 749 | 830 | 822 | 820 | 412 | 663 | 716 | 540 | 763 | 757 | 805 | 624 |
| 6 | 720 | 637 | 802 | 883 | 899 | 873 | 465 | 717 | 767 | 591 | 816 | 810 | 858 | 678 |
| 7 | 773 | 714 | 855 | 936 | 976 | 926 | 518 | 771 | 818 | 642 | 869 | 863 | 911 | 732 |
| 8 | 826 | 791 | 908 | 989 | 1053 | 979 | 571 | 825 | 869 | 693 | 922 | 916 | 964 | 786 |
| 9 | 879 | 868 | 961 | 1042 | 1130 | 1032 | 624 | 879 | 920 | 744 | 975 | 969 | 1017 | 840 |
| 10 | 932 | 945 | 1014 | 1095 | 1207 | 1085 | 677 | 933 | 971 | 795 | 1028 | 1022 | 1070 | 894 |
| 11 | 985 | 1022 | 1067 | 1148 | 1284 | 1138 | 730 | 987 | 1022 | 846 | 1081 | 1075 | 1123 | 948 |
| 12 | 1038 | 1099 | 1120 | 1201 | 1361 | 1191 | 783 | 1041 | 1073 | 897 | 1134 | 1128 | 1176 | 1002 |
| 13 | 1091 | 1176 | 1173 | 1254 | 1438 | 1244 | 836 | 1095 | 1124 | 948 | 1187 | 1181 | 1229 | 1056 |
| 14 | 1144 | 1253 | 1226 | 1307 | 1515 | 1297 | 889 | 1149 | 1175 | 999 | 1240 | 1234 | 1282 | 1110 |
| 15 | 1197 | 1330 | 1279 | 1360 | 1592 | 1350 | 942 | 1203 | 1226 | 1050 | 1293 | 1287 | 1335 | 1164 |

1with oligonucleotides used in Supply et al., 2000, Mol. Microbiol., 36, 762-771 and described in Mazars et al 2001, PNAS, 98, 1901-1906 (see supplemental data on the PNAS Web site http://www.pnas.org)

2with oligonucleotides used in Supply et al. 2001, J. Clin. Microbiol., 39, 3563-3571

7.3.3 Background reading and references

7.3.3.1 Genotyping M. tuberculosis

The insertion sequence IS6110 has been shown to be an important contributor to the generation of genetic diversity in Mtb. IS6110 may insert into genes, disrupting the coding sequence, generate genomic deletions though recombination events and affect gene expression through its intrinsic promoter activity.

For many years, our appreciation of the phylogenetic relationships between strains of Mtb has been restricted by the use of limited measures of genetic variation. The standard genotyping instrument for Mtb has been IS6110 restriction fragment length polymorphism (RFLP) analysis. This is an excellent tool for epidemiological investigations since it is highly discriminatory, but is not necessarily best suited for phylogenetic analysis. The insertion sequence has irregular rates of transposition and the difficulty in standardizing methodology and interpretation of the fingerprints obtained has lead to a confusing proliferation of nomenclature compounded by a lack of communication between centres.

More recently, typing methods such as spoligotyping and multiple interspersed repetitive unit (MIRU) analysis have offered a more easily standardized approach, albeit not necessarily more suited to phylogenetic analysis.

Spoligotyping is based on polymorphism in the direct repeat region of the Mtb chromosome. At this locus, there is a series of repeat 36 base pair sequences which are interspersed by non-repetitive spacer elements each 35 to 41 base pairs in length. The presence or absence of each of 43 of these spacer regions is detected using a reverse dot blot technique. Briefly, the spacer regions are amplified using primers directed at the common flanking direct repeat elements. The amplified fragments are then hybridized to a membrane that has probes for each of the spacer elements. The membrane is washed and the presence of bound fragments detected by chemiluminescence. This technique is technically simple, reproducible and easily coded and has, as a result, allowed the development of an international database and introduced a degree of consistency to the naming of the major lineages of Mtb. Moreover, there is a high degree of consistency in the identification of the major lineages of Mtb between spoligotyping, large sequence polymorphism and single nucleotide polymorphism analysis. However, since major changes in spoligotype may occur due to single deletion events, it is not always useful for defining the relationship between strains, nor is it sufficiently discriminatory to differentiate closely related strains. It is therefore less useful for epidemiological investigations where it is important to differentiate between closely related but different strains.

The genome of Mtb contains repetitive 40-100 base pair sequence elements known as ‘mycobacterial interspersed repetitive units’ (MIRU) which are found as tandem repeats at multiple intergenic loci throughout the chromosome. These loci demonstrate hypervariability in the number of repeats, leading to the use of the term “variable number of tandem repeats” (VNTR).

The typing method MIRU-VNTR is based on the detection of the number of tandem repeats present at several different loci. The most commonly used methods examine 12-20 of these loci. MIRU-VNTR is performed by amplifying each locus and determining the size of the amplicon and hence the number of repeats at each locus. These are then combined into a multi-digit code. This technique is more discriminatory than spoligotyping, and, since many independent loci are assessed, may be more appropriate for phylogenetic analysis.  However it is less visually intuitive than spoligotyping, more labour intensive and does not always concord well with SNP-based phylogenetic analysis. This may relate to the potential for gain or loss of repeats at each locus, leading to the possibility of convergent evolution (broadly different strains acquiring similar numbers of repeats at a particular locus).

One of the important mechanisms by which genetic diversity arises in Mtb is through large chromosomal deletions. Such deletions, termed large sequence polymorphisms (LSP), have been used to classify strains of Mtb into distinct lineages. Since recombination is rare, all progeny of a parent strain will carry the same deletion. The presence or absence of these deletions is used to define major lineages within Mtb. This approach infers a basic structure to the evolutionary tree by assuming the sequential acquisition of deletions. It is also useful for establishing clonal relationships amongst strains with different spoligotype or RFLP patterns. This technique has been used to show that those strains of Mtb which have been sequenced cluster in two related ‘modern’ lineages and do not represent the global diversity of strains.

The most robust method, however, for defining the global phylogeny of Mtb, is the use of SNP analysis and whole genome sequencing. SNP analysis, in a restricted form, was used to classify Mtb strains into three principal genetic groups, based on SNPs in codon 463 of the katG gene and codon 95 of the gyrA gene (Sreevatsan et al 1997). This three-branched tree defined the broad phylogeny of Mtb for many years. The results of recent more extensive SNP analysis will advance this understanding. Gagneux and colleagues have sequenced 90 genes in 104 globally representative strains to define the relationships between the major lineages of Mtb. This is an important step forward in defining the nature of diversity in Mtb and offers an ideal starting point for evaluating the clinical implications of such diversity. The application of relatively low cost, high throughput sequencing technology to multiple representative strains of Mtb may significantly advance our understanding of the extent of genetic diversity in tuberculosis as well as the relationships between strains. Figure 7 and Table 2 summarize the major methods used for genotyping Mtb strains.

Finally, the region of the Mtb chromosome containing the so-called PPE/PE gene families has been shown to be an important source of genetic variation. In particular, it has been suggested that this region may be partly responsible for antigenic variation in Mtb, which could feasibly be a mechanism for immune evasion.

7.4 Whole genome sequencing. Analysis of antibiotic resistance from whole genome sequence data of Mycobacterium tuberculosis clinical isolates

Introduction

Although molecular genotyping techniques have proved useful in confirming the occurrence of re-infection, they provide a surrogate measure of inter strain diversity. Recent applications of whole genome sequencing to M. tuberculosis have demonstrated that this technique can provide greater insight into the evolution, epidemiology and mutation rate of this clonal organism.

Tuberculosis patients undertake treatment consisting of combination of four antimicrobial drugs (isoniazid, rifampicin, pyrazinamide and ethambutol) and the duration of the treatment is 6 months (2HRZE/4HR). Antimicrobial resistance in M. tuberculosis usually arises as a result of point mutation in the coding sequence leading to structural changes in the target proteins.

Principle

This guide will help you identify mutations conferring antibiotic resistance in whole genome sequences (WGS) of M. tuberculosis clinical isolates. You will be provided WGS data of TB isolates from patients treated with multiple antibiotics. We will use Artemis, a genome browser tool, to analyze the sequences of four genes rpoB, katG, inhA and gyrA, which encode for ribosomal polymerase  subunit, catalase/peroxidase, enoyl reductase and DNA gyrase, respectively. The mutations in these genes are responsible for antibiotic resistance to rifampicin, isoniazid and fluoroquinolones.

The total genomic DNA of M. tuberculosis isolates was sequenced by next generation sequencing on MiSeq (Illumina) using paired-end reads sequencing approach. Thousands of short reads were acquired for each sample. You can find these sequences in each patient’s file under the names SRR974841_1.fastq.gz and SRR974841_2.fastq.gz. There are two files for each sample as the library was constructed by paired-end sequencing. You can unzip these files and explore the contents.

The sequences of clinical isolates are mapped against a reference sequence - Mycobacterium tuberculosis H37Rv with Bowtie2 software creating BAM files. The BAM files are located within the folders for each patient. SNP calling was performed with Samtools mpileup with a threshold of 13 for base quality. The result is VCF format files containing all variations that are detected in each patient’s isolate. The BAM and VCF are large data files. We will use Artemis software to analyze the resulting sequences.

Artemis is a free genome browser and annotation tool and can be downloaded from Sanger Institute website (http://www.sanger.ac.uk/resources/software/artemis/).

The aim of this practical is to identify any mutations present in any of the four genes rpoB, katG, inhA and gyrA.

Procedure

- Open Artemis software.

- Open Mycobacterium tuberculosis H37Rv genome.

  - In the Artemis window “File->Open-> path(…./H37RvReference /sequence.gb)”

  - Locate rpoB gene “Goto->Navigator” and type “rpoB” in the “Goto Feature with Gene Name”.

  - You will be directed to the rpoB sequence, you can explore the basis and the amino acid sequences in the bottom window.

  - Select the rpoB gene and press the “V” key to get more information about this gene.

- Open the data from one patient, for example patient1.

  - In the genome window “File->Read BAM/VCF..” and select the “mapping/P1/SRR974841_P1.bam”.

  - Add more to the list: open “mapping/P1/ SRR974841_P1.vcf.gz”. Press OK.

  - VERY IMPORTANT: open the file with “vcf.gz” extension, not just the “.vcf”.

  - Go to the rpoB gene and identify if patient 1 has any mutations in this gene.

- Open the genome sequences of all four patients together and analyze the mutations in the genes rpoB, katG, inhA and gyrA. Create a results table with all mutations that you can find in all four genes for every patient.

  - Note: You can select one BAM or VCF file when you have multiple files loaded in Artemis. Right click in the reads or SNP area and select “BAM Files” or “VCF Files” and check/uncheck what file you want or do not want to view. This is important because if you analyze more than one uploaded sequence in the Read Map Area you will not be able to identify a particular mutation. All reads are mixed from all uploaded sequences.

Overview

In a long-term outbreak (1997 to 2010) WGS analyses of the 86 isolates revealed 85 single nucleotide polymorphisms (SNPs), subdividing the outbreak into seven genome clusters (two to 24 isolates each), plus 36 unique SNP profiles. Genome-based clustering patterns mapped contact tracing data and the geographical distribution of the cases than clustering patterns based on classical genotyping. A maximum of three SNPs were identified in eight confirmed human-to-human transmission chains, involving 31 patients.  Multiple analyses support an average mutation rate of ~0.3 SNPs per genome per year. Another study sequenced 390 separate isolates from 254 patients, including representatives from all five major lineages of M tuberculosis. The estimated rate of change in DNA sequences was 0.5 single SNPs per genome per year (95% CI 0.3–0.7) in longitudinal isolates from 30 individuals and 25 families. Divergence is rarely higher than five SNPs in three years. 109 (96%) of 114 paired isolates from individuals and households differed by five or fewer SNPs. More than five SNPs separated isolates from none of 69 epidemiologically linked patients, two (15%) of 13 possibly linked patients, and 13 (17%) of 75 epidemiologically unlinked patients (three-way comparison exact p<0·0001).   Thus, it is usually assumed that SNP differences of less than 6 are associated with recent transmission.

Whole genome sequencing is of value in investigating the relationship of a relapse strain with the initial culture for a patient failing on or after treatment.  A recent study demonstrated that relapse strains had less than 6 SNPs whereas strains that were designated as reinfections had >1300 SNPs and belonged to different lineages making it possible to distinguish these outcomes unequivocally2.  An important finding of this paper was that mixed infection was found to be common in the patients in South Africa.  This may have important implications for future investigations in the evolution of antibiotic resistance and strain transmission.

Figure 7.4.1 Schematic representation of the M. tuberculosis genome, indicating the genetic basis of genotyping techniques. The circular chromosome of the reference strain H37Rv is shown together with examples of the major genetic elements used for strain genotyping. For clarity only one MIRU (mycobacterial interspersed repetitive unit) locus, one IS6110 (insertion sequence), one region of difference (RD) and one single nucleotide polymorphism are shown.

Table 7.4.1 Comparison of techniques for molecular typing or evaluating genetic diversity within M. tuberculosis

|  | IS6110 RFLP* | Spoligotyping | MIRU-VNTR† | LSP analysis‡ | SNP§ analysis | Whole Genome sequencing |
| --- | --- | --- | --- | --- | --- | --- |
| Discriminatory power | Excellent | Fair to poor | Good to excellent (newer protocols) | Poor | Poor, likely to improve with increased SNP identification | Very high |
| Ease of use | Time consuming and technically demanding; requires extracted chromosomal DNA | Rapid and simple; can be performed directly on specimens or heat-killed cultures. | Rapid and fairly simple; automation requires access to sophisticated equipment; can be performed on heat-killed cultures. | Simple and robust | Rapid and fairly simple; high throughput analysis requires access to sophisticated equipment | This method has potential for rapid use but is still too cumbersome for routine clinical use. |
| Interpretation | Simple, but not easily standardized | Simple visual interpretation | Simple visual interpretation | Straightforward | Straightforward | Requires extensive bio-informatics support |
| Data sharing | Complex, lack of standardized nomenclature | Straightforward, standardized binary or octal coding | Straightforward, standardized numerical coding | Straightforward, standardized nomenclature | Straightforward, standardized nomenclature | Simple |
| Utility for epidemiological investigations | Excellent due to high discriminatory power | Useful for rapid cluster identification but requires secondary confirmation | Excellent and rapid with newer protocols | Poor due to low discriminatory power | Poor at present due to low discriminatory power | Very high. Able to dissect transmission series and emergence of resistance simultaneously |
| Utility for phylogenetic analysis | May be limited by irregular rates of transposition and favoured sites. | Fairly good correlation with SNP-based phylogeny. Large deletions may bias analysis | Relatively poor correlation with SNP-based phylogeny | Useful for evolutionary history (sequential deletions) and for identifying major lineages | Gold-standard; low rate of SNP in Mtb necessitates large-scale sequencing to identify informative SNPs | Gold standard once the practical problems of bioinformatics is addressed |

Table footnote:

* restriction fragment length polymorphism

† mycobacterial interspersed repetitive unit-variable number of tandem repeat

‡ large sequence polymorphism

§ single nucleotide polymorphism

Additional reading for IS6110/MIRU /Sequencing

Almeida Da Silva PE, Palomino JC. Molecular basis and mechanisms of drug resistance in Mycobacterium tuberculosis: classical and new drugs. J Antimicrob Chemother. 2011 Jul;66(7):1417-30.

Arnold C*, Westland L, Mowat G, Underwood A, Magee J, Gharbia S (2005) Single nucleotide polymorphism-based differentiation and drug resistance detection in Mycobacterium tuberculosis from isolates or directly from sputum. Clin Microbiol Infect.

Borrell S, Thorne N, Español M, Mortimer C, Orcau A, Coll P, Gharbia S, González-Martín J, Arnold C. Tuberculosis (Edinb). (2009) Comparison of four-colour IS6110-fAFLP with the classic IS6110-RFLP on the ability to detect recent transmission in the city of Barcelona, Spain.May;89(3):233-7.

Bryant JM, Harris SR, Parkhill J, et al. Whole-genome sequencing to establish relapse or re-infection with Mycobacterium tuberculosis: a retrospective observational study. Lancet Respiratory Medicine doi:10.1016/S2213-2600(13)70231-5.

Comas I, Homolka S, Niemann S and Gagneux S (2009) Genotyping of genetically monomorphic bacteria: DNA sequencing in Mycobacterium tuberculosis highlights the limitations of current methodologies. PloS ONE 2009;4:e7815

Frothingham, R. and W. Meeker-O'Connell (1998). "Genetic diversity in the Mycobacterium tuberculosis complex based on variable numbers of tandem DNA repeats." Microbiology 144: 1189-1196.

Goyal, M., N. Saunders, et al. (1997). "Differentiation of Mycobacterium tuberculosis isolates by spoligotyping and IS6110 restriction fragment length polymorphism." J Clin Microbiol 34: 647-651.

Kanduma E, McHugh TD, Gillespie SH. Molecular methods for Mycobacterium tuberculosis strain typing: a users guide.  J Appl Microbiol. 2003;94(5):781-91

# Maguire H, Dale JW, McHugh TD, Butcher PD, Gillespie SH, Costetsos A, Al-Ghusein H, Holland R, Dickens A, Marston L, Wilson P, Pitman R, Strachan D, Drobniewski FA, Banerjee DK. Molecular epidemiology of tuberculosis in London 1995-7 showing low rate of active transmission.Thorax. 2002 Jul;57(7):617-22.

McHugh TD, Batt SL, Shorten RJ, Gosling RD, Uiso L, Gillespie SH. Mycobacterium tuberculosis lineage: a naming of the parts.  Tuberculosis (Edinb). 2005 May;85(3):127-36.

Maguire H, Dale JW, McHugh TD, Butcher PD, Gillespie SH, Costetsos A, Al-Ghusein H, Holland R, Dickens A, Marston L, Wilson P, Pitman R, Strachan D, Drobniewski FA, Banerjee DK.  Molecular epidemiology of tuberculosis in London 1995-7 showing low rate of active transmission. Thorax. 2002 Jul;57(7):617-22.

Otal I, Samper S, Asensio MP, Vitoria MA, Rubio MC, Gomez-Lus R, Martin C (1997). Use of a PCR method based on IS6110 polymorphism for typing Mycobacterium tuberculosis strains from BACTEC cultures. J Clin Microbiol. Jan;35(1):273-7.

Ramaswamy S, Musser JM (1998) Molecular genetic basis of antimicrobial agent resistance in Mycobacterium tuberculosis: 1998 update. Tuber Lung Dis: 79(1):3-29.

Reisig F, Kremer K, Amthor B, van Soolingen D, Haas WH (2005). Fast ligation-mediated PCR, a fast and reliable method for IS6110-based typing of Mycobacterium tuberculosis complex.  J Clin Microbiol. Nov;43(11):5622-7.

Roetzer A, Diel R, Kohl TA, et al. Whole genome sequencing versus traditional genotyping for investigation of a Mycobacterium tuberculosis outbreak: a longitudinal molecular epidemiological study. PLoS Med 2013; 10: e1001387.

Ruddy M, McHugh TD, Dale JW, Banerjee D, Maguire H, Wilson P, Drobniewski F, Butcher P, Gillespie SH.Estimation of the rate of unrecognized cross-contamination with mycobacterium tuberculosis in London microbiology laboratories.  J Clin Microbiol. 2002 Nov;40(11):4100-4.

Shorten RJ, McGregor AC, Platt S, Jenkins C, Lipman MC, Gillespie SH, Charalambous BM, McHugh TD. When is an outbreak not an outbreak? Fit, divergent strains of Mycobacterium tuberculosis display independent evolution of drug resistance in a large London outbreak. J Antimicrob Chemother. 2013 Mar;68(3):543-9.

Sreevatsan S, Pan X, Stockbauer K, et al. (1997) Restricted gene polymorphism in the Mycobacterium tuberculosis complex indicates evolutionarily recent global dissemination. Proc Natl Acad Sci USA;94:9869-9874

Supply, P., E. Mazars, et al. (2000). Variable human minisatellite-like regions in the Mycobacterium tuberculosis genome. Mol Microbiol 36: 762-771.

Supply P, Allix C, Lesjean S, Cardoso-Oelemann M, Rüsch-Gerdes S, Willery E, Savine E, de Haas P, van Deutekom H, Roring S, Bifani P, Kurepina N, Kreiswirth B, Sola C, Rastogi N, Vatin V, Gutierrez MC, Fauville M, Niemann S, Skuce R, Kremer K, Locht C, van Soolingen D (2006). Proposal for standardization of optimised mycobacterial interspersed repetitive unit-variable number tandem repeat typing of Mycobacterium tuberculosis. 2006  J Clin Microbiol. 44:4498-4510.

Thorne N, Evans JT, Smith EG, Hawkey PM, Gharbia S, Arnold C (2007). An IS6110-targeting fluorescent amplified fragment length polymorphism alternative to IS6110 restriction fragment length polymorphism analysis for Mycobacterium tuberculosis DNA fingerprinting. Clin Microbiol Infect. Oct;13(10):964-70.

Thorne N, Borrell S, Evans J, Magee J, García de Viedma D, Bishop C, Gonzalez-Martin J, Gharbia S, Arnold C (2011) IS6110-based global phylogeny of Mycobacterium tuberculosis. Infect Genet Evol. Jan;11(1):132-8.

van Embden JD, Cave MD, Crawford JT, Dale JW, Eisenach KD, Gicquel B, Hermans P, Martin C, McAdam R and Shinnick TM (1993). Strain identification of Mycobacterium tuberculosis by DNA fingerprinting: recommendations for a standardized methodology. J Clin Microbiol, , 406-409, Vol 31, No. 2.

Walker TM, Ip CL, Harrell RH, et al. Whole-genome sequencing to delineate Mycobacterium tuberculosis outbreaks: a retrospective observational study. The Lancet Infectious Diseases 2012. doi:10.1016/S1473-3099(12)70277-3.

Wilson SM, Goss S, Drobniewski F (1998). Evaluation of strategies for molecular fingerprinting for use in the routine work of a Mycobacterium reference unit. J Clin Microbiol. Nov;36(11):3385-8.

Appendix 1: E-Gel product information

E-Gel® Electrophoresis System – taken from Invitrogen E-Gel® Technical Guide

Introduction

The E-Gel® agarose gel electrophoresis system is a complete bufferless system for agarose gel electrophoresis of DNA samples. The major components of the system are:

• E-Gel® pre-cast agarose gels

• Electrophoresis bases

E-Gel® pre-cast agarose gels are self-contained gels that include electrodes packaged inside a dry, disposable, UV-transparent cassette. The E-Gel® agarose gels run in a specially designed device that is a base and power supply combined into one device (two bases are available for running E-Gels, the new iBase™ system and the original, economical E-Gel® Powerbase™).

Advantages of E-Gel® Agarose Gels

Using E-Gel® agarose gels for electrophoresis of DNA samples offer the following advantages:

• Provides fast, safe, consistent, high-resolution electrophoresis

• Eliminates the need to prepare agarose gels and buffers, and to stain gels

• Compatible with most commercially available robotic systems for high-throughput agarose gel electrophoresis

• Available in a variety of agarose percentages, well formats, and throughput capacities to suit your applications

• Offered with a number of different DNA gel stains to accommodate your application

• Includes E-Gel® CloneWell™ and E-Gel® SizeSelect™ gels, to accelerate and simplify DNA gel purification and improve cloning results

Throughput Capacity

Three categories of E-Gel® agarose electrophoresis systems are available from Invitrogen based on your throughput requirements.

• Low-Throughput E-Gel® Electrophoresis System designed for electrophoresis of 8–16 DNA samples per gel.

• Medium-Throughput E-Gel® Electrophoresis System designed for electrophoresis of 48 DNA samples per gel. This system is compatible for use with multichannel pipettors or automated liquid handling systems.

• High-Throughput E-Gel® Electrophoresis System is designed for electrophoresis of 96 DNA samples per gel. This system is compatible for use with multichannel pipettors or automated liquid handling systems.

E-Gel® Single Comb and DoubleComb Gels

The E-Gel® single comb and double comb gels are bufferless gels containing electrodes embedded in the agarose matrix. Each gel contains an ion generating system (TAE buffer system), a pH balancing system, and ethidium bromide for DNA staining and is packaged inside an UV-transparent cassette. To create a patented bufferless system, each E-Gel® single comb and double comb cassette contains two ion exchange matrices (IEMs) that are in contact with the gel and electrodes. The IEMs supply a continuous flow of ions through out the gel resulting in a sustained electric field required for running the gel (see figure below).

Appendix 2: Agarose Gel Electrophoresis Sizing of PCR products

# Overview of A.G.E.

Electrophoresis is a technique that results in the separation of charged molecules based on their size. DNA is a negatively charged molecule in solution so it will move to the positive pole in an electric field, in this case through a matrix of polymerised agarose molecules.  All linear DNA has the same charge per unit length and linear pieces migrate according to size.

The end result is that large pieces of DNA move slower than small pieces of DNA. The position that the DNA migrated to in the gel is observable under ultraviolet light when the gel is stained with ethidium bromide.  Sizes of your DNA samples can be determined by running a size standard (or a DNA molecular weight ladder) alongside your samples. A variation of agarose gel electrophoresis, called pulsed-field gel electrophoresis, makes it possible to separate even extremely long DNA molecules. Ordinary gel electrophoresis fails to separate such molecules because the steady electric field stretches them out so that they travel end-first through the gel in snakelike configurations at a rate that is independent of their length. In pulsed-field gel electrophoresis, by contrast, the direction of the electric field is changed periodically, which forces the molecules to reorient before continuing to move through the gel. This reorientation takes much more time for larger molecules, so that progressively longer molecules move more and more slowly. As a consequence, even entire bacterial or yeast chromosomes separate into discrete bands in pulsed-field gels and so can be sorted and identified on the basis of their size.

# Protocol

# Components of AGE:

Agarose

Casting tray and comb

Electrophoresis buffer (usually 1xTBE)

# Power pack

Electrophoresis tank

Stain (0.5mg/ml Ethidium bromide)

Transilluminator

Several different buffers have been recommended for electrophoresis of DNA. The most commonly used for duplex DNA are TAE (Tris-acetate-EDTA) and TBE (Tris-borate-EDTA). DNA fragments will migrate at somewhat different rates in these two buffers due to differences in ionic strength. Buffers not only establish a pH, but provide ions to support conductivity.

DNA Loading Buffer (6x)

6mM EDTA 
300mM NaOH 
18% Ficoll (in water) 
0.15% Bromophenol blue
0.25% Xylene Cyanol

# Making a 2% agarose gel:

- Weigh out 2 g of agarose in a large conical flask and add 100 ml 1xTBE. Stopper the flask with a foam bung.

- Microwave for 2 or 3 minutes until completely dissolved.

- Cool under a cold tap for a short time to 60°C (70°C for concentrations 2% or above).

- Meanwhile, prepare a casting tray by sealing the ends, if necessary, and finding an appropriate comb for the tray you are using and the number of samples you are loading.

- Pour molten agarose into the casting tray, careful not to make any bubbles in the gel.

- Leave gel to set for approx. 30 mins.

Table to show DNA Size Separation by Agarose Concentration

| Effective Range of Separation for Linear Fragments (kb) | Agarose (%) |
| --- | --- |
| 30 to 1 | 0.5 |
| 12 to 0.8 | 0.7 |
| 10 to 0.5 | 1.0 |
| 7 to 0.4 | 1.2 |
| 3 to 0.2 | 1.5 |

# N.B. Bromophenol blue (BP) generally co-migrates with 0.5 kb fragments of DNA.

# Loading samples:

- Once set, remove comb carefully and remove tape from the ends if used.

- Place the casting tray with gel into the electrophoresis tank and make sure there is sufficient TBE buffer in the tank to completely submerge the gel.  The orientation of the gel should be so the wells are closest to the negative electrode. Load your PCR products with loading buffer and include a DNA ladder in one of the wells. Bromophenol blue (BP) generally co-migrates with 0.5 kb fragments of DNA. Stain with 0.5 mg/ml ethidium bromide for 10 minutes. Place lid on electrophoresis tank.

- Switch on the power pack. Voltage: Agarose gels can be run at various voltages, depending on the separation desired and the available time. As the voltage applied to a gel is increased, larger fragments migrate proportionally faster than small fragments. For that reason, the best resolution of fragments larger than about 2 kb is attained by applying no more than 5 volts per cm to the gel (the cm value is the distance between the two electrodes, not the length of the gel). For PCR products smaller than 600 bp, separation is better and bands are sharper if gels are run very fast (3-4 hours for a 15-20 cm long 2-3% agarose gel). When the same gel runs at a low voltage overnight (14-16 hours) the products become less separable due to the diffusion in the gel.

# Staining and viewing gel:

- When finished, switch off the power pack and remove lid from tank.

- Take out casting tray with gel inside and slide gel out from the tray.

- Carefully place the gel into the 0.5 mg/ml ethidium bromide in 1 X TBE tank to stain (wearing NITRILE gloves!) for 2-15mins, depending on the age of the stain.

- Place stained gel under UV light.